Specifications text

Article number

242377

Type

NSFP-20000DLC

Short description

SFP Module, 1.25 Gbps, 20KM, Duplex LC, Tx/Rx at 1310 nm, hot-plug, SFP MSA-compliant, Digital Monitoring

Main Features

1250 Mbps data rate on 9/125 µm SMF

Dual-fiber design (separate TX and RX fibers)

Up to 20 km range (single-mode fiber)

LC optical connector

Compliant with SFP MSA & SFF-8472

Integrated Digital Diagnostics Monitoring (DDM)

Wide temp. range: -40°C to +85°C (industrial)

Single 3.3V power supply

Metal housing for low EMI interference

TX fault and Loss of Signal (LOS) alerts

Low power consumption (< 300 mA)

RoHS compliant and lead-free

Other Features (Specifications)

| Dimensions (HxWxD) | 14,2x12,5x56,3mm |
| --- | --- |
| Type | SFP-Modul |
| Fiber optic connector | Duplex LC |
| Transmission medium | optical fiber |
| Model | module |
| Number of fibers | 2 |
| Wavelength | 1310nm |
| Range | 20 km |
| Operating temperature | -40°C ~ +85°C |
| Weight | 17,05g |
| Manufacturer order number | 242377 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services