Specifications text

Article number

245151

Type

ISM-52F0021WMB

Short description

1/2,8" Network Module Camera, 1920x1080, Day&Night,, WDR, 2,1mm, H.265, PoE, 12VDC

Main Features

1/2,8" 2.0 MP CMOS-Sensor

Resolution max. 1920x1080 (Full HD)

Board Camera 50x50x50mm

Quad Streaming (H.265, H.264, MJPEG)

True Day & Night (TDN)

Noise Reduction (2D-/3D-DNR)

WDR 120dB, Smart BLC

Slot for Micro SD memory card

ONVIF conformance, PoE IEEE802.af, Cross Web-Browsing

Audio-, Alarm- In- and Outputs

Privacy zones, password protection, Cross Web-Browsing

NDAA compliant

Other Features (Specifications)

| Audio support | Yes |
| --- | --- |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Vandalism-resistant | No |
| Alarm inputs | 1 |
| Resolution standard | 2 MP |
| Recording sensor | CMOS, Sensor SC500AI |
| Lighting | no |
| Image resolution max. | 1920x1080 |
| Image transfer rate max. | 25 fps |
| Horizontal angle of view | 138 |
| Focal length | 2,1 |
| Chip size | 1/2,8" |
| Dimensions (WxHxD) (extended) | 50x50x50mm |
| Alarm processing | Recording internal storage (SD card),E-Mail,FTP,HTTP,HTTPS,alarm out |
| Audio | 2 Way Audio |
| Resolution (extended) | 1920x1080,1280x720,720x480,640x480,352x288 |
| Watch (62px/m) | 6.1 m |
| Picture size | 16:9 |
| Aperture Range (F) | F1.8 |
| Browser | Firefox, Google Chrome, MS Internet Explorer 10/11, Safari |
| Data stream types | Quad Stream |
| Detect (125px/m) | 2.9 m |
| ESC | 1~1/20.000Sek. |
| External connections | Alarm output,Alarm input,Audio input/output,RJ-45,12VDC input (DC plug) |
| Flickerless function | Yes |
| Identify (250px/m) | 2.1 m |
| Camera shape | Printed circuit board |
| Scope of delivery | manual,Connection cable set |
| Manual Electric Shutter (MES) | 1~1-20,000sec. |
| Menu languages | Chinese, English, French, German, Japanese, Korean, Russian |
| Password | Administrator and User |
| Privacy Zone | 4 Programmable Zone |
| SDK | upon request |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 20.5 m |
| White balance | Auto,manual |
| Housing type | without housing |
| Internal memory | microSD card slot |
| Camera Type | Board Camera |
| Photosensitivity | 0.03 Lux bei F1.8 |
| MOD (Minimum Object Distance) | 0.3 m |
| Mounting type | Built-in module |
| Network Protocols | DHCP, DNA, DVRNS, HTTP, HTTPS, ICMP, IGMP, IPv4, NTP, RTCP, RTP, RTSP, SMTP, SSL, TCP, UDP, Zeroconf |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 245151 |
| Power over Ethernet | IEEE 802.3af |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | 0°C ~ +40°C |
| Video Compression | H.264, H.265, MJPEG |
| Wide Dynamic Range (WDR) | TWDR 120dB |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |