Specifications text

Article number

243990

Type

INB-68M2812MJBA

Short description

Network Camera Bullet incl. junction box 3840x2160, Day/Night, WDR, AI 2,8-12mm, IR, WL, 12VDC, PoE, IP67

Main Features

1/2.8" Sony STARVIS CMOS sensor, 8 MP/30 fps

AI analysis functions including object classification

Integrated connection box for quick and clean installation

WDR 120 dB, 3D-DNR noise reduction

Motorised varifocal lens 2.8–12mm

Smart Codec H.265/H.264, multi-streaming

Browser-independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/M/G

Local storage via micro SD card

Supports audio and external alarm input and output

NDAA compliance

IR and white light LED illumination, IP67

Other Features (Specifications)

| Audio support | Yes |
| --- | --- |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Sunshade canopy | Yes |
| Integrated wall mount | Yes |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Alarm inputs | 1 |
| Resolution standard | 8 MP |
| Recording sensor | Chipset NT98529, CMOS, Sony IMX415 |
| Lighting | Infrared |
| Image resolution max. | 3840x2160 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 107 - 34° |
| Focal length | 2,8 - 12 mm |
| Chip size | 1/2,8" |
| Dimensions (WxHxD) (extended) | 135,2x353,8mm |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,alarm out,Cloud Upload |
| Audio | 1/1 Line Input and Output, 2 Way Audio |
| Resolution (extended) | 3840x2160 (16:9),2592x1944 (4:3),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x960 (4:3),1280x720 (16:9),640x480 (4:3),320x240 (4:3) |
| Illumination range | Approx. 35m IR,approx. 30m Warm Light |
| Remark | The face detection and license plate recognition functions are only available with an inserted SD card or in conjunction with a recorder from the eneo IN series. To guarantee you an optimum user experience,we recommend that you only use components of the |
| Watch (62px/m) | 24.8 m |
| Picture size | 16:9, 4:3 |
| Browser | Edge, Firefox, Google Chrome, MS Internet Explorer 10/11, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 11.3 m |
| ESC | 1/5 ~ 1/20.000sec. |
| External connections | RJ-45,Alarm in/output,Audio input/output,micro SD card slot 256 GB |
| Flickerless function | Yes |
| Identify (250px/m) | 4.1 m |
| Camera shape | Bullet |
| Scope of delivery | manual,bag of screws |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Spanish, Turkish |
| Privacy Zone | Areas-ON/OFF switchable |
| SDK | upon request |
| Day/night mode | Yes |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 63.3 m |
| White balance | Automatic,Manual |
| Color (Housing) | RAL9003,Pantone P 179-1 |
| Housing type | Outside |
| Housing material | Aluminium |
| Internal memory | microSD card slot |
| Camera Type | Face, License plate recognition |
| Photosensitivity | 0.007 Lux bei F1.0 |
| MOD (Minimum Object Distance) | 1.5 m |
| Mounting type | Ceiling, Wall |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens Type | Varifocal (motorized) |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 243990 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -35°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Video Outputs | no |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Zoomfactor (optical) | 4 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| --- | --- |
| NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC |
| NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |