Specifications text

Article number

244619

Type

eneo VMD-FIHD22PC0A

Short description

22" (56cm) LCD Monitor, FHD, 1920x1080, LED, HDMI, DisplayPort

Main Features

Professional 22"(56 cm) LCD monitor

Maximum resolution Full HD 1920x1080 pixels

LED Back Light

Digital 3D filtering and noise reduction

Built to run 24x7x365 continuously

Different video inputs HDMI,DisplayPort

Built-in speakers

Plastic cabinet including stand

Other Features (Specifications)

| Other |  |
| --- | --- |
| EAN/GTIN | 4260229840979 |
| Comb filter | Yes |
| Applications | 24/7 |
| Audio Inputs | No |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 56 cm |
| Screen diagonal (inches) | 22 " |
| Dimensions (WxHxD) (extended) | 494,00x286,30x41,26mm |
| Resolution (extended) | 800x600,1024x768,1280x1024,1600x900,1920x1080 |
| Fortification | VESA Standard 100x100mm |
| Remark | NDAA compliance under Section 889 |
| Display | Hard coating (3H), Haze: 1% |
| Weight | 3.5 kg |
| Scope of delivery | manual,Power supply unit 100 ~ 240VAC / 12VDC,50/60Hz,power cord,feet,HDMI cable |
| Menu languages | English, French, German, Italian, Japanese, Spanish |
| Pixel error class | ISO 13406-2, Class 2 |
| Transmission Technology | Display Port (DP), HDMI 2.0 |
| Energy efficiency class | E |
| Color (Housing) | Black |
| Housing material | Plastic |
| Brightness (typical) | 240 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 1000:1 |
| Power consumption | 25 W |
| Maximum Image Resolution (HV) | 1920×1080 |
| Manufacturer order number | 244619 |
| Response time (typical) | 5 ms |
| Aspect ratio | 16:9 |
| Series | eneo Monitor |
| Power supply | 12VDC |
| Video Inputs | DP, HDMI |
| Brand | eneo |

Additional Productinformation

Related products

| eneo VM-LCDWMB2 | Wall bracket for LCD monitors horizontal adjustable, support up to max. 8 kg |
| --- | --- |
| eneo VM-PFF | Privacy filter foils for 22" LCD-Monitors |
| eneo VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |
| jetrics VM-UCWM20 | Compact slim tilt wall mount LCD Monitors, 23" to 42", VESA support up to 25Kg max. |
| jetrics VM-UMA10 | Aluminium gas spring monitor arm for wall mount, LCD Monitors, max. 27" support up to 7 kg max. |
| jetrics VM-UTM10 | Universal Table Mount for LCD Monitors, up to 45", VESA support up to 13Kg max. |
| jetrics VM-UWM10 | Universal Wall Mount for LCD Monitors, up to max. 27", VESA support up to 15 kg max. |
| jetrics VM-UWM30 | Universal Wall Mount for LCD Monitors, up to 43", VESA support up to 20Kg max. |

Recommendations

| jetrics jetcon HDMI Kabel 1 |  |
| --- | --- |
| jetrics jetcon HDMI Kabel 3 |  |
| jetrics jetcon HDMI Kabel 5 |  |

Services