Specifications text

Article number

244621

Type

eneo VMD-FI4K27PC0A

Short description

27" (69cm) LCD Monitor, 4K UHD, 3840x2160, LED, USB, DisplayPort, 3xHDMI

Main Features

Professional 27"(69cm) LCD monitor

Maximum resolution 4K UHD 3840x2160 pixels

LED Back Light

High brightness, High contrast

Perfect presentation through digital 3D filtering

and noise reduction

Built to run 24x7x365 continuously

Different video inputs DisplayPort, 3x HDMI

Built-in speakers

Including stand

Plastic housing

Other Features (Specifications)

| Other |  |
| --- | --- |
| EAN/GTIN | 4260229840931 |
| Comb filter | Yes |
| Applications | 24/7 |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 69 cm |
| Screen diagonal (inches) | 27 " |
| Dimensions (WxHxD) (extended) | 615,80x364,50x190,00mm |
| Resolution (extended) | 800x600,1024x768,1280x1024,1600x900,1920x1080,3840x2160 |
| Fortification | VESA Standard 100x100mm |
| Remark | NDAA compliance under Section 889 |
| Display | Hard coating (3H), Haze: 25% |
| Weight | 7 kg |
| Scope of delivery | Manual,Power cord,feet,HDMI cable |
| Menu languages | English, French, German, Italian, Spanish |
| Pixel error class | ISO 13406-2, Class 2 |
| Transmission Technology | Display Port (DP), HDMI 2.0 |
| Energy efficiency class | F |
| Color (Housing) | Black |
| Housing material | Plastic |
| Brightness (typical) | 240 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 1000:1 |
| Power consumption | 40 W |
| Maximum Image Resolution (HV) | 3840× 2160 |
| Manufacturer order number | 244621 |
| Response time (typical) | 5 ms |
| Aspect ratio | 16:9 |
| Series | eneo Monitor |
| Power supply | 110-240VAC |
| Video Inputs | 3x HDMI 2.0, DP |
| Brand | eneo |

Additional Productinformation

Related products

| eneo VM-LCDWMB2 | Wall bracket for LCD monitors horizontal adjustable, support up to max. 8 kg |
| --- | --- |
| eneo VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |
| Jetrics VM-UCWM20 | Compact slim tilt wall mount LCD Monitors, 23" to 42", VESA support up to 25Kg max. |
| Jetrics VM-UMA10 | Aluminium gas spring monitor arm for wall mount, LCD Monitors, max. 27" support up to 7 kg max. |
| Jetrics VM-UTM10 | Universal Table Mount for LCD Monitors, up to 45", VESA support up to 13Kg max. |
| Jetrics VM-UWM10 | Universal Wall Mount for LCD Monitors, up to max. 27", VESA support up to 15 kg max. |
| Jetrics VM-UWM30 | Universal Wall Mount for LCD Monitors, up to 43", VESA support up to 20Kg max. |

Recommendations

| Jetrics jetcon HDMI Kabel 1 |  |
| --- | --- |
| Jetrics jetcon HDMI Kabel 3 |  |
| Jetrics jetcon HDMI Kabel 5 |  |

Services