Specifications text

Article number

246620

Type

eneo IND-45F0022M0B

Short description

Mini Network Dome Camera Fix, 2880x1620, Day/Night, WDR, AI 2,2mm, IR, 12VDC, PoE, IP66

Main Features

1/2.7" CMOS sensor, maximum resolution 5 MP/30fps

AI analysis functions including object classification

WDR 120dB, 3D-DNR noise reduction

Fixed focal length lens 2.2mm (HFoV 156°)

Smart Codec H.265/H.264, multi-streaming

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M

Multilingual menu navigation (OSD)

Local storage via Micro SD card

Supports audio (built-in microphone)

NDAA compliance

IR LED illumination, IP66

Other Features (Specifications)

| Other |  |
| --- | --- |
| EAN/GTIN | 4260229840900 |
| Analysis | Crowds (CD), Enter Region, Face, Heat Map (HM), Intrusion, Leave Region, License plate recognition (LPR), Line crossing (LCD), Object classification, Object Classification Human / Vehicle (PD&VD), Passage count (CC), Queues (QD), Rare Sounds (RSD) |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Audio compression | G.711 |
| Resolution standard | 5 MP |
| Recording sensor | Chipset SSC30KQ, CMOS, SC533HAI |
| Lighting | Infrared |
| Image resolution max. | 2880x1620 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 156° |
| Vertical angle of view | 81° |
| Focal length | 2,2 mm |
| Chip size | 1/2,7" |
| Dimensions (WxHxD) (extended) | 64,3x115,3mm |
| Alarm processing | Recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,Cloud Upload |
| Audio | 1 Way Audio, Built-in microphone |
| Resolution (extended) | 2880x1620 (16:9),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x720 (16:9),640x480 (4:3),320x240 (4:3) |
| Illumination range | Approx. 20m |
| Remark | Face and license plate recognition require an SD card or an eneo IN recorder. For optimal results,use only eneo components. For mixed brands,we offer a compatibility check for a fee (info: item #229235 at VIDEOR) |
| Watch (62px/m) | 5,1m |
| Aperture Range (F) | F2.0 |
| Browser | Edge, Firefox, Google Chrome, MS Internet Explorer 10/11, Safari |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Detect (125px/m) | 2,3m |
| ESC | 1/5 ~ 1/20.000sec. |
| External connections | RJ-45,micro SD card slot 256 GB |
| Flickerless function | Yes |
| Weight | 0.3 kg |
| Identify (250px/m) | 1,3m |
| Camera shape | Minsters |
| Scope of delivery | Quick installation guide,drilling template,3x wall plugs with screws,1x Torx wrench,1x rubber gasket,1x snagless RJ45 boot |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| SDK | Upon request |
| Security | Complicated password, IP address filtering |
| Signal-to-noise ratio | 43 dB |
| Day/night mode | Automatic,Day,Night,Image,Time |
| Text overlay | Switchable on/off |
| Perceiving (25px/m) | 14,6m |
| White balance | Automatic,Manual |
| Ethernet interface | 100Base-TX, 10Base-T |
| Color (Housing) | RAL9003,Other colors available upon request |
| Color (dome) | clear |
| Housing type | Outside |
| Housing material | Aluminium, Plastic |
| Internal memory | microSD card slot |
| Camera Type | Domecamera |
| Power consumption | 5 W |
| Photosensitivity | 0.004lux bei F1.6(AGC ON) |
| MOD (Minimum Object Distance) | 0.2 m |
| Mounting type | Ceiling, Wall |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, NTP, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 246620 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP66, NEMA 4X |
| Series | eneo IN-Serie |
| Control Interfaces | Ethernet |
| Power supply | Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -35°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Video Compression | H.264, H.264+, H.265, H.265+, MJPEG |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| jetrics jetcat6SFTP-GN-0,5/1 |  |
| jetrics jetcat6SFTP-GN-1/1 |  |
| jetrics jetcat6SFTP-GN-2/1 |  |
| jetrics jetcat6SFTP-GN-3/1 |  |
| jetrics jetcat6SFTP-GN-7,5/1 |  |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE fix dome camera | in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |