Specifications text

Article number

217871

Type

eneo ISD-55P0010P0A

Short description

1/1.8" Network Dome, Fixed, Day&Night, 2592x1944, 360°, H.264, 1.05mm, Indoor

Main Features

1/1.8" Sony STARVIS CMOS-Sensor

1.05mm fixed lens FishEye camera

360° Panorama view

Integration in eneo Center Software

True Day&Night

Resolution max. 2048x1536@15fps

Compression H.264/MJPEG, Web Casting

Cross Web-Browsing

D-WDR (Digital Wide Dynamic Range)

Framerate increase on motion detection

OpenVPN support

Compact design for indoor application

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Lighting | No |
| Dimensions (WxHxD) (extended) | 63,8x119mm |
| Weight | 0.27 kg |
| SDK | upon request |
| Text overlay | available |
| Color (Housing) | white |
| Housing type | Inside |
| Housing material | Plastic |
| Camera Type | Fisheye Camera |
| Power consumption | 5.3 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile S |
| Power over Ethernet | IEEE 802.3af |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | No |
| Resolution standard | 5 MP |
| Recording sensor | CMOS |
| Chip size | 1/1,8" |
| Camera shape | Minsters |
| Day/night mode | Automatic,Day,Night |
| White balance | Automatic,fluorescent,Outdoor,Manual,Light bulb |
| Photosensitivity | 0,3 Lux bei F2,8 |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 2592x1944 |
| DORI |  |
| Watch (62px/m) | 14 m |
| Detect (125px/m) | 6 m |
| Identify (250px/m) | 4 m |
| Perceiving (25px/m) | 36 m |
| Functions |  |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | recording internal storage (SD card),E-Mail,FTP,Boost,HTTP,TCP,UDP,HTTPS,XML,alarm out |
| Audio | 2 Way Audio |
| Browser | Edge: MJPEG setup limited, Google Chrome: MJPEG setup limited, Mozilla: MJPEG setup limited, Safari: MJPEG setup limited |
| Menu languages | Chinese, English, French, German, Japanese, Korean, Russian |
| Internal memory | microSDHC card, microSDXC card |
| Notes |  |
| Remark | To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necessary,we offer a compatibility check |
| Interfaces / Inputs/ Outputs |  |
| Alarm inputs | 1 |
| External connections | Alarm output,Alarm input,Audio input/output,RJ-45,12VDC input (DC plug),connection cable approx.40cm |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Lens |  |
| Horizontal angle of view | 360° |
| Vertical angle of view | 360° |
| Focal length | 1,05 mm |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 15 fps |
| Resolution (extended) | 2592x1944 (4:3),2048x1536 (4:3),1944x1944,1440x1440,1280x1024 (5:4),1280x960 (4:3),1024x1024,1024x768 (4:3),720x720,704x576 (4:3),640x480 (4:3) |
| Data stream types | 2x H.264,1x MJPEG simultan |
| Video Compression | H.264, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 217871 |
| Series | eneo SN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services