Specifications text

Article number

212715

Type

IPP-82A0030MIA

Short description

1/2,8" Network Dome, 1920x1080, PTZ, Day&Night, 4,3-129mm, Infrared, IP66

Main Features

1/2.8" Sony CMOS-Sensor

30x AF Zoom, 4,3~129mm / Digital zoom

Full HD 1920x1080 pixels, 1080p@50/60fps

Compression H.264/MJPEG, Web Casting

Digital noise reduction (DNR)

VCA Video Analytic

Zoom synchronized & fixed IR LEDs

Real time WDR (Wide Dynamic Range) 25/30fps

DIS (Digital Image Stabilization)

Privacy zones

Alarm in- and outputs, micro SD card slot

High-PoE midspan included

Other Features (Specifications)

| Dimensions (HxWxD) | 315x201.38mm |
| --- | --- |
| Series | eneo SN-Series |
| Resolution Standard | HD 1080p |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Video compression | H.264, M-JPEG |
| Image resolution max. | 1920x1080 |
| Image Transmission Rate | 60 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,35 Lux at F1,6, 50 IRE |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Lens Type | auto focus zoom |
| Focal Length | 4,3 - 129,0 mm |
| Iris Control | DC |
| Zoom Factor | 30x |
| Digital Zoom | 16x |
| Horizontal Angle of View | 61,8 - 2,11° |
| Vertical Angle of View | 36,4 - 1,29° |
| MOD (Mininum Object Distance) | 1 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone Cool Gray 1C |
| Hosuing material | Aluminium, Plastic |
| Heater | no |
| Rotation range | 360° |
| Speed (manual) | 380°/Sek. |
| Alarm inputs | 4 |
| Alarm outputs | 1 |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSDHC card, microSDXC card |
| Analysis functions | Defog, Field detection, Gesichtserkennung, Motion detection, Tampering protection, Trip wire plus object counting |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 28W |
| Power over Ethernet | IEEE 802.3at |
| Vandal proof | no |
| Operating temperature | -30°C ~ 65°C |
| ONVIF | ONVIF Profile S |
| Protection rating | IP66 |
| Camera type | PTZ |
| Audio | 2 Way Audio |
| Remote configuration | Browser |
| Illumination range | approx. 300m |
| Weight | 4.9kg |
| Manufacturer order number | 212715 |
| Brand | eneo |

Additional Productinformation

Related products

| EDC-CMT1 | Ceiling Mount Tube with Flange Plate for PTZ Dome Cameras, Gray |
| --- | --- |
| EDC-WMB1 | Wall Mount for PTZ Dome Cameras with Mounting Kit, Gray |
| eneo Site Manager | Management Software for eneo IP products |
| EVX-900B | Media Converter, indoor, Transmitter-Receiver, 1x RJ45, 1x BNC, 12V, up to 1000m |
| EVX-900R | Media Converter, Indoor, Transmitter-Receiver, 1x RJ45, 1x RJ-11, 12V, up to1000m |
| IAD-5SG1004MUA | Gigabit Switch, Unmanaged, 60W, 4x PoE, 2x RJ45, 10/100/1000 Mbps, Desktop |
| IAD-5SG1008MUA | Gigabit Switch, Unmanaged, 120W, 8x PoE, 1xSFP, 1x RJ45, 10/100/1000 Mbps, Desktop |
| IAM-5SE1008MUA | Ethernet Switch, 8xPoE 100Mbps 1x RJ45, and 1x SFP 1000Mbps, Unmanaged, Industrial Design |
| IAM-5SG1008MMA | Gigabit Switch, Managed, 240W, 8x PoE, 4x SFP, 1000Mbps, Industrial Design, -40°C~+75°C |
| IAR-7SE1024MMA | Gigabit Switch, 2x RJ45, 24x PoE 100Mbps, 2xSFP 1000Mbps, 390W, Managed, 19", 1U |
| IAR-7SG1008MMA | Gigabit Switch, Managed, 135W, 8x PoE, 2x SFP, 2xRJ45, 10/100/1000 Mbps, 19", 1 U |
| IAR-7SG1016MMA | Gigabit Switch, Managed, 270W, 16x PoE, 4x SFP, 10/100/1000 Mbps, 19", 1HU |
| IAR-7SG1024MMA | Gigabit Switch, Managed, 390W, 24x PoE, 4x SFP, 10/100/1000 Mbps, 19", 1HU |

Recommendations

| CON-KAM-IPKON | IP preconfiguration network Camera, functional test, Firmware update |
| --- | --- |
| LACKIERUNG PTZ oder Panoramakamera | Extra charge for special paint PTZ dome or panoramic camera in RAL color |
| SanDisk High Endurance 32GB | MicroSDHC Speicherkarte, 32GB, Class 10, 20MB/s, für Videoüberwachung optimiert |
| SanDisk High Endurance 64GB | MicroSDXC Speicherkarte, 64GB Class 10, 20MB/s, Videoüberwachung optimiert |
| WDD064G1P0A | MicroSDXC Speicherkarte, 64GB, Class 10, UHS Class 1, 50MB/s, für Videoüberwachung |

Services