Specifications text

Article number

220593

Type

IAD-5SE1008MUA

Short description

Switch, Unmanaged, 120W 8x PoE, 2 x RJ45 Uplink 10/100/1000 Mbps, Desktop

Main Features

Ethernet PoE Switch, desktop application

Gigabit Uplink Ports: 2 x RJ45

8 x 10/100 PoE Ports IEEE 802.3af/at

Up to 30W per Port, Total ?120W

Surge protection IEC61000-4-5 (6KV)

CCTV mode allows transmission up to 250m @ 10Mbps

Plug and Play no further settings necessary

Other Features (Specifications)

| Dimensions (HxWxD) | 27x200x101,8mm |
| --- | --- |
| Type | Unmanaged |
| Series | eneo Switch |
| Mounting | desktop |
| Blower | no |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | no |
| Ports | 2x 10/100/1000Base-T, 8x 10/100Base-TX |
| PoE ports (number) | 8 |
| Management | no |
| Model | Desktop |
| Supply voltage | 48~57 VDC |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 120 W |
| Operating temperature | -10°C ~ +55°C |
| Weight | 500g |
| Manufacturer order number | 220593 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services