Specifications text

Article number

220593

Type

eneo IAD-5SE1008MUA

Short description

Switch, Unmanaged, 120W 8x PoE, 2 x RJ45 Uplink 10/100/1000 Mbps, Desktop

Main Features

Ethernet PoE Switch, desktop application

Gigabit Uplink Ports: 2 x RJ45

8 x 10/100 PoE Ports IEEE 802.3af/at

Up to 30W per Port, Total ≤120W

Surge protection IEC61000-4-5 (6KV)

CCTV mode allows transmission up to 250m @ 10Mbps

Plug and Play no further settings necessary

Other Features (Specifications)

| General |  |
| --- | --- |
| Number of PoE ports | 8 |
| Design | Desktop |
| Dimensions (WxHxD) (extended) | 27x200x101,8mm |
| Weight | 500 kg |
| Type | Unmanaged |
| Mounting type | Desktop |
| PoE Budget (Total) | 120 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -10°C ~ +55°C |
| Notes |  |
| Remark | CCTV mode: transmission of 10Mbps @ 250m,CCTV mode,PoE Power output: 29.2W @ 1m; 25.8W @ 100m; 24.2W @ 150m; 19.9W @ 250m |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Ports | 2x 10/100/1000Base-T, 8x 10/100Base-TX |
| Parts supplied |  |
| Scope of delivery | manual,PSU |
| Network |  |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX |
| Security | CCTV mode |
| Internal Switching Capacity (Gbit) | 5.6 Gbps |
| Internal Switching Capacity (MP per second) | 4.2 Mpps |
| Product labeling: |  |
| Manufacturer order number | 220593 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services