Specifications text

Article number

216472

Type

IAD-5SG1008MUA

Short description

Gigabit Switch, Unmanaged, 120W, 8x PoE, 1xSFP, 1x RJ45, 10/100/1000 Mbps, Desktop

Main Features

Gigabit PoE Switch, desktop application

Gigabit Uplink Ports: 1x RJ45, 1x SFP

8x 10/100/1000 PoE Ports, IEEE 802.3af/at

Up to 30W per Port, Total ≤120W

ESD protection IEC61000-4-2 Standard

CCTV mode allows transmission up to 250m @ 10Mbps

Brackets and 150W power-adaptor scope of supply

Other Features (Specifications)

| Number of PoE ports | 8 |
| --- | --- |
| Design | Desktop |
| Dimensions (WxHxD) (extended) | 27x200x101,8mm |
| Remark | CCTV mode: transmission of 10Mbps @ 250m,CCTV mode,PoE Power output: 29.2W @ 1m; 25.8W @ 100m; 24.2W @ 150m; 19.9W @ 250m |
| Weight | 500 kg |
| Scope of delivery | manual,power cord,Mounting Bracket,PSU |
| Multicast | No |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | CCTV mode |
| Internal Switching Capacity (Gbit) | 20 Gbps |
| Internal Switching Capacity (MP per second) | 14.9 Mpps |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Type | Layer2 Switch |
| Fan | No |
| Management | No |
| Mounting type | Desktop |
| Manufacturer order number | 216472 |
| PoE Budget (Total) | 120 W |
| Ports | 1x 10/100/1000Base-TX, 1x 1000Base-X SFP, 8x 10/100/1000Base-T |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Rack installation | No |
| Series | eneo Switch |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -10°C ~ +45°C |
| VLAN | No |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services