Specifications text

Article number

218734

Type

IAR-7SH1024MMA

Short description

Gigabit Switch, Managed, 390W, 4x SFP 10G,  24x PoE, 10/100/1000 Mbps, 19", 1U

Main Features

PoE Switch, Rack Mounting

10Gigabit Uplink Ports: 4x SFP

24x 10/100/1000 PoE Ports, IEEE 802.3af/at

Up to 30W per port, total ≤390W

Fast Ring-Redundant, Reconfiguration

ESD protection IEC61000-4-5 standard

Managed L3 switch, Built in low-noise fans

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password

Temperature range -10° C ~ +50° C

Rack mounting brackets included

Other Features (Specifications)

| Manufacturer order number | 218734 |
| --- | --- |
| Brand | eneo |

Additional Productinformation

Related products

| RJT-1000SFP | RJ45- SFP Transceiver Modul, 1000Base-T, 0°~ +70°C |
| --- | --- |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services