Specifications text

Article number

219526

Type

eneo IAM-5SG1004MMA

Short description

Gigabit Switch, Managed, 120W, 4x PoE, 2x SFP, 1000Mbps, Industrial Design, -40°C~+75°C

Main Features

Gigabit PoE Switch, DIN Rail

2x SFP Gigabit Uplink Ports

4x 10/100/1000 PoE Ports, IEEE 802.3af/at

Supports VLAN, RTSP, "Fast Ring", and QoS

Up to 30W per port, Total ≤120W

Fast Ring-Redundant, Reconfiguration

Supports redundant power supply connection

ESD protection IEC61000-4-5 Standard

Web configuration, Web management

Fault Relay, Console Port, Status LED

PoE reset button for each port

CCTV mode allows transmission up to 250m @ 10Mbps

Other Features (Specifications)

| General |  |
| --- | --- |
| Management | Yes |
| Number of PoE ports | 4 |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 46,5x110x163mm |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 680 kg |
| Type | Layer2 Switch |
| Mounting type | DIN Rail, Wall |
| PoE Budget (Total) | 120 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -40°C ~ +75°C |
| Functions |  |
| Multicast | Yes |
| Notes |  |
| Remark | External power supply specification: 100~240VAC,150W,30x70x150mm,-30°~+70°,CCTV mode: transmission of 10Mbps @ 250m,CCTV mode,PoE Power output: 29.2W @ 1m; 25.8W @ 100m; 24.2W @ 150m; 19.9W @ 250m |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Ports | 2x 1000Base-X SFP, 4x 10/100/1000Base-T |
| Parts supplied |  |
| Scope of delivery | manual,power cord,Mounting Bracket,PSU |
| Network |  |
| VLAN | Yes |
| Eneo _netzwerkstandards | IEEE 802.1D Spanning Tree Protocol, IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | Fast Ring, Password, Port Isolation, Port Speed Setting, Radius Authentication |
| Internal Switching Capacity (Gbit) | 24 Gbps |
| Internal Switching Capacity (MP per second) | 8.9 Mpps |
| Product labeling: |  |
| Manufacturer order number | 219526 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

| eneo SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| --- | --- |
| eneo SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| eneo SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| eneo SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| eneo SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| Jetrics NDR-240-48 DIN RAIL | Power Supply, 230VAC / 48VDC, max. 240W, for DIN rail mounting |
| --- | --- |

Services