Specifications text

Article number

216474

Type

IAR-7SG1016MMA

Short description

Gigabit Switch, Managed, 270W, 16x PoE, 4x SFP, 10/100/1000 Mbps, 19", 1HU

Main Features

PoE Switch, Rack Montage

Gigabit Uplink Ports: 4x SFP

16x 10/100/1000 PoE Ports, IEEE 802.3af/at

Up to 30W per port, total ?270W

Fast Ring-Redundant, Reconfiguration <20ms

ESD protection IEC61000-4-5 standard

Managed L2 switch

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password

Temperature range -10° C ~ +50° C

Rack mounting brackets included

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Manufacturer order number | 216474 |

Additional Productinformation

Related products

| RJT-1000SFP | RJ45- SFP Transceiver Modul, 1000Base-T, 0°~ +70°C |
| --- | --- |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services