Specifications text

Article number

219809

Type

MPP-72A0020MIA

Short description

1/2,8" HD Analogue Dome, PTZ, Day&Night, 20x, 4,7-94mm, WDR 1920x1080, Infrared, IP66

Main Features

1/2,8" Sony STARVIS CMOS-Sensor

20x Auto Focus Zoom, 4.7 ~ 94mm / 16x Digital

Resolution max. 1920x1080 (Full HD)

Separate outputs for HD signals and composite

Signal formats: HD-TVI, AHD selectable, composite

Real time WDR (Wide Dynamic Range) 25 / 30fps

Integrated infrared lighting (up to 60m)

3D DNR noise reduction, auto-calibration

Protected areas, Defog function

Alarm inputs and outputs, 12VDC, IP66

Other Features (Specifications)

| General |  |
| --- | --- |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 208,3x182,4mm |
| Illumination range | ca. 60m |
| Text overlay | available |
| Housing type | Outside |
| Housing material | Aluminium |
| Mounting type | Ceiling, Wall |
| Protection | IP66 |
| Power supply | 12VDC |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Zoomfactor (optical) | 20 |
| Camera |  |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | PTZ |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night |
| White balance | Automatic,Manual,Light bulb,Fluorescent light,Outdoor |
| Photosensitivity | 0,35 Lux bei F1,6,50 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| DORI |  |
| Watch (62px/m) | 28 m |
| Identify (250px/m) | 7 m |
| Perceiving (25px/m) | 70 m |
| Functions |  |
| Digital Noise Reduction (DNR) | Yes |
| Turning range | 360 ° |
| Home Position | Multiple actions adjustable |
| Menu languages | English, French, German, Italian, Polish, Portuguese, Spanish |
| Tours | 8 selectable tours |
| Speed (manual control) | 380 °/sec |
| Speed (Target Approach) | 380 ° |
| Interfaces / Inputs/ Outputs |  |
| Alarm inputs | 4 |
| Number of alarm outputs | 2 |
| External connections | Alarm in/output,voltage input,RS-485,Composite (FBAS),HD-TVI / AHD BNC,connection cable approx.25cm |
| Interface Protocols | Fastrax IIIE, Pelco D, UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol), RS-485 |
| Parts supplied |  |
| Scope of delivery | manual,plug set,bag of screws |
| Lens |  |
| Horizontal angle of view | 55,5 - 3° |
| Focal length | 4,7 - 94 mm |
| Aperture Range (F) | F1.6~F3.0 |
| Lens Type | Auto Focus Zoom |
| Zoom Factor (Digital) | 16 |
| Video |  |
| Signal Format | AHD @ 2MP, Composite (FBAS), HD-TVI @ 2MP |
| Product labeling: |  |
| Manufacturer order number | 219809 |
| Series | eneo COAXIZE |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services