Specifications text

Article number

217646

Type

MSM-42F0037MPA

Short description

1/2.8" HD Board Camera, Colour, 1920x1080, 3.7mm, Pinhole, WDR, Housing, 12VDC

Main Features

1/2,8" 2 Megapixel SONY STARVIS CMOS-Sensor

Board Camera with rugged metal cabinet

Resolution max. 1920x1080 (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Pinhole lens 3.7mm

Electronic Day/Night function

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

Other Features (Specifications)

| General |  |
| --- | --- |
| Dimensions (WxHxD) (extended) | 37x37,8x21,5mm |
| Text overlay | available |
| Housing type | Inside |
| Housing material | Metal |
| Camera Type | Board Camera |
| Mounting type | Ceiling, Wall |
| Power supply | 12VDC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 113dB |
| Camera |  |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Module |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | not available |
| White balance | Automatic,Preset,Manual |
| Photosensitivity | 0,08 Lux bei F1,4,40 IRE |
| System | Color |
| Day/night switching | no |
| DORI |  |
| Watch (62px/m) | 21 m |
| Identify (250px/m) | 5 m |
| Perceiving (25px/m) | 59 m |
| Functions |  |
| Digital Noise Reduction (DNR) | Yes |
| Menu languages | Chinese, English, German, Japanese, Korean |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| External connections | voltage input,1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC) |
| Interface Protocols | UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol) |
| Parts supplied |  |
| Scope of delivery | manual,mounting accessories |
| Lens |  |
| Horizontal angle of view | 82° |
| Focal length | 3,7 mm |
| Lens Type | Prime lens |
| Video |  |
| Signal Format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP |
| Product labeling: |  |
| Manufacturer order number | 217646 |
| Series | eneo COAXIZE |
| Brand | eneo |

Additional Productinformation

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

Services

| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |
| --- | --- |