Specifications text

Article number

218771

Type

MPD-64A0003P0A

Short description

1/2,9" HD Dome, Fixed 2560x1440, Day&Night, AF Zoom, WDR, 3,2-9mm, Infrared, Indoor

Main Features

1/2,9" 6,64 Megapixel SONY STARVIS CMOS-Sensor

Resolution max. 2560x1440@25/30fps

Optical 3x auto focus zoom, 3,2~9mm

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD selectable, Composite SUB

Digital signal: EX-SDI, HD-SDI selectable

DOL-WDR by triple scan images at 30/25fps &

ROI (Region of Interest) WDR mode

Point of Interest & Smart Motion Zoom

Wedge adapter included (10° inclination)

Scratch resistant hard coated bubble

Easy installation, 3-Axis gimbal

Other Features (Specifications)

| Audio support | No |
| --- | --- |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Motion detector | No |
| Horizontal angle of view | 75 - 26° |
| Focal length | 3,2 - 9 mm |
| Chip size | 1/2,9" |
| Digital Noise Reduction (DNR) | Yes |
| Turning range | 0 ° |
| Dimensions (WxHxD) (extended) | 94x116mm |
| Illumination range | ca. 30m |
| Remark | NDAA compliance under Section 889 |
| Watch (62px/m) | 32 m |
| Aperture Range (F) | F1.6 |
| External connections | voltage input,1x SDI Signal (BNC),1x HD Analog Signal (BNC),connection cable approx.15cm |
| Flickerless function | Yes |
| Identify (250px/m) | 6.6 m |
| Camera shape | Minsters |
| Scope of delivery | manual,Video sub-out cable,bag of screws |
| Menu languages | Chinese, English, German, Japanese, Korean |
| Multicast | No |
| Interface Protocols | UTC (HD Signal Format), UTC (HD Signal Format) |
| Signal-to-noise ratio | 50 dB |
| Signal Format | AHD @ 4MP, EX-SDI @ 4MP, HD-SDI @ 2MP, HD-TVI @ 4MP |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night,night-color mode |
| Text overlay | available |
| Vertical tilt range | 65 °(25 ~ 90°) |
| Perceiving (25px/m) | 80 m |
| Integrated wall mount | No |
| White balance | AUTO,AUTOext,Manual,Preset |
| Color (dome) | clear |
| Backlight compensation | No |
| Housing type | Inside |
| Housing material | Plastic |
| Heating | Yes |
| Photosensitivity | 0,1 Lux bei F1,6,40 IRE |
| Low Speed Shutter | No |
| Fan | No |
| Mounting type | Ceiling, Wall |
| Lens Type | Auto Focus Zoom |
| Manufacturer order number | 218771 |
| Series | eneo KX-Serie,eneo COAXIZE,eneo Callisto Indoor |
| Control Interfaces | Coaxitron (HD Protocol) |
| Power supply | 12VDC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Vandalism-resistant | No |
| Wide Dynamic Range (WDR) | D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, WDR 113dB |
| Zoomfactor (optical) | 3 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services