Specifications text

Article number

218772

Type

MPD-74A0003M0A

Short description

1/2,9" HD Dome, Fixed 2560x1440, Day&Night, AF Zoom, WDR, 3,2-9mm, Infrared, IP67

Main Features

1/2,9" 6,64 Megapixel SONY STARVIS CMOS-Sensor

Resolution max. 2560x1440@25/30fps

Optical 3x auto focus zoom, 3,2~9mm

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD selectable, Composite SUB

Digital signal: EX-SDI, HD-SDI selectable

DOL-WDR by triple scan images at 30/25fps &

ROI (Region of Interest) WDR mode

Point of Interest & Smart Motion Zoom

IP67, built-in heater, 3-Axis gimbal

For wall or ceiling flush mount

Hard coated bubble, Anti-scratch protection

Other Features (Specifications)

| Dimensions (HxWxD) | 119x140mm |
| --- | --- |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Series | eneo Callisto, eneo COAXIZE, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 2560x1440 |
| Resolution Pixels | 1280x720, 1920x1080, 2560x1440 |
| System | True Day&Night |
| Sensor size | 1/2.9" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,1 Lux at F1,6, 40 IRE |
| Low Speed Shutter | yes |
| Digital Noise Reduction | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR, WDR 113dB |
| Motion Detector | yes |
| Lens Type | auto focus zoom |
| Focal Length | 3,2 - 9 mm |
| Iris Control | DC |
| Zoom Factor | 3x |
| Digital Zoom | yes |
| Observe (62 px/m) | 32 - 88 m |
| Recognize (125 px/m) | 14 - 43 m |
| Identify (250 px/m) | 6,6 - 21 m |
| Detect (25px/m) | 80 - 215 m |
| Horizontal Angle of View | 75 - 26° |
| Vertical Angle of View | 41 - 15° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone Cool Gray 1C |
| Color (bubble) | clear |
| Hosuing material | Aluminium |
| Heater | yes |
| Blower | no |
| Rotation range | +/-173° |
| Video outputs | AHD, composite (CVBS) SUB, EX-SDI, HD-SDI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog, Digital image stabilization |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, 24VAC |
| Power consumption | 8W |
| Vandal proof | yes |
| Operating temperature | -20°C ~ +50°C |
| Certifications | IK10 |
| Protection rating | IP67 |
| Signal format | AHD @ 4MP, EX-SDI @ 4MP, HD-SDI (1.485 Gbps), HD-TVI @ 4MP |
| Camera type | Dome |
| Frame rate | 25/30fps @ 1920x1080p, 25/30fps @ 2560x1440p, 50/60fps @ 1280x720p |
| Aperture range (F) | F1.6 |
| Illumination range | approx. 40m |
| Weight | 1.0kg |
| Manufacturer order number | 218772 |
| Brand | eneo |

Additional Productinformation

<h2>Modularer eneo HD Analog Dome für die Überwachung im Außenbereich</h2><p>Parkplätze, Zufahrten, Lieferzonen, Freigelände und andere Außenbereiche überwacht der HD Analog Dome MPD-74A0003M0A effektiv und zuverlässig. Der IK10-geschützte eneo HD Analog Dome aus der Callisto-Serie ist mit einem soliden Aluminiumgehäuse sowie einer kratzfesten Kuppel ausgestattet und für den Einsatz bei Temperaturen von -20°C bis 50°C ausgelegt. Qualitativ hochwertige Videobilder mit Auflösungen von bis zu 2560 x 1440 Pixeln stellen der leistungsstarke Sony Starvis CMOS-Sensor (6,64 Megapixel) und das Autofokus-Zoom-Objektiv (3facher optischer Zoom) mit einer Brennweite 3,2?9mm sicher. Zudem unterstützt der HD Analog Dome mit HD-TVI, AHD, Composite SUB, EX-SDI und HD-SDI mehrere analoge und digitale Videosignale. <br><br></p><h3>Intelligente Zoom-Funktionen</h3><p>Die Überprüfung von Aktivitäten im Überwachungsbereich wird durch die Funktionen Smart Motion Zoom und Pointing Zoom zusätzlich vereinfacht. Detektiert der HD Analog Dome z. B. Bewegungen von Objekten oder Personen in vorab definierten Bildbereichen, führt Smart Motion Zoom einen optischen Schwenk aus und holt die relevanten Bildregionen näher heran. Die Empfindlichkeit der Bewegungserkennung und die Zoomdauer können Anwender individuell konfigurieren. Mit der Pointing-Zoom-Funktion können Anwender während der Live-Ansicht eine Zoomfahrt auf einen ebenfalls vorab definierten Bildbereich auslösen. Und mittels Preset-Zoom sind bis zu vier Zoom-Positionen einstellbar. <br><br></p><h3>Konstant hohe Bildqualität 24/7</h3><p>Zwei High Power LEDs (850nm) mit einer Beleuchtungsreichweite von rund 40m sorgen bei Dunkelheit für den situativen Überblick. Die Bildoptimierungsfunktionen DOL-WDR, BLC / HLC Gegenlichtkompensation, digitale Bildstabilisierung und Defog stellen dabei sicher, dass der HD Analog Dome bei Tag wie bei Nacht hochwertige Videobilder generiert. In Kameratests erreichte der HD Analog Dome die folgenden DORI-Distanzen: Wahrnehmen (25 px/m) auf 80 bis 215m, Beobachten (62 px/m) auf 32 bis 88m, Erkennen (125 px/m) auf 14 bis 43m und Identifizieren (250 px/m) auf 6,6 bis 21m. <br><br></p><h3>Zubehör für die errichterfreundliche Installation</h3><p>Für die professionelle und errichterfreundliche Montage kann der modulare eneo HD Analog Dome mit praktischem Installationszubehör ergänzt werden. So kann die Kamera mit einem Montagering, Keiladapter, Wandarm oder einer robusten Anschlussbox kombiniert und an die Gegebenheiten vor Ort angepasst werden. </p>

Related products

| AK-5 | Camera Junction Box, for Callisto Dome series, IP66 |
| --- | --- |
| DOME-S1 | Sun Shield for eneo Callisto Dome Series |
| DOME-WB1+DOME/ Basis1 | Wall Bracket & Surface mount base, Callisto Dome line |
| DOME/BASIS1 | Surface mount base , Callisto Dome line |
| DOME/WEDGE2 | Wedge Adapter, Callisto Dome Line |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| PS12DC-11 | Plug-in Power Supply Unit, 100-240VAC/12VDC (1,0A), regulated, with DC Plug |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |