Specifications text

Article number

217718

Type

MPB-72M0550M0A

Short description

1/2,8" HD Camera, Day&Night, 1920x1080, Infrared, WDR, 5-50mm, 12/24V, IP67

Main Features

1/2,8" 2 Megapixel SONY STARVIS 2 CMOS-Sensor

Super low light performance by STARVIS 2 sensor

Resolution max. 1920x1080 (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized lens 5 ~ 50mm

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

IP67, built heater

Easy installation, central locking screw

IR illumination switchable via OSD menu

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 100x107,8x277mm |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Series | eneo Candid Medium, eneo COAXIZE, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,08 Lux at F1,4, 40 IRE |
| Digital Noise Reduction | yes |
| Lens Type | Varifocal (motorized) |
| Focal Length | 5 - 50 mm |
| Iris Control | DC |
| Zoom Factor | 10x |
| Digital Zoom | no |
| Observe (62 px/m) | 39 - 235 m |
| Recognize (125 px/m) | 19 - 120 m |
| Identify (250 px/m) | 9 - 60 m |
| Detect (25px/m) | 100 - 600 m |
| Horizontal Angle of View | 45 - 7° |
| Vertical Angle of View | 25 - 4° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone Cool Gray 1C |
| Hosuing material | Aluminium |
| Heater | yes |
| Video outputs | AHD, Composite (CVBS), CVI, EX-SDI, HD-SDI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, 24VAC |
| Power consumption | 10.5W |
| Vandal proof | no |
| Operating temperature | -30°C ~ +50°C |
| Protection rating | IP67 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 113dB |
| Motion Detector | yes |
| Signal format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, EX-SDI (270 Mbps), HD-SDI (1.485 Gbps), HD-TVI @ 2MP |
| Camera type | Bullet |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Aperture range (F) | F1.6 |
| Illumination range | approx. 60m |
| Weight | 1.35kg |
| Manufacturer order number | 217718 |

Additional Productinformation

<h2>HD Analog Bullet Kamera mit hoher optischer Leistungsfähigkeit</h2><p>Die HD Analog Bullet Kamera MCB-72M2712M0A ist mit einem motorisierten Varifokalobjektiv mit einer Brennweite von 5 bis 50mm ausgestattet. Dadurch hat die Full HD Kamera eine sehr hohe optische Leistungsfähigkeit, wie die im Test ermittelten DORI-Distanzen belegen: Wahrnehmen (25 px/m) auf 100 bis 600m, Beobachten (62 px/m) auf 39 bis 235m, Erkennen (125 px/m) auf 19 bis 120m und Identifizieren (250 px/m) auf 9 bis 60m. Dabei erreicht die eneo mit einem Sony CMOS Sensor bestückte HD Analog Bullet Kamera eine maximale Bildauflösung von 1920x1080 Pixeln.<br><br></p><h3>Problemlose Integration in Bestandssystemen</h3><p>Die HD Analog Bullet Kamera unterstützt die analogen Videosignale HD-TVI, AHD, CVI und Composite sowie digitalen Formate EX-SDI und HD-SDI. Dies erlaubt ohne Weiteres eine Einbindung der Full HD Kamera in bestehende, Koax-basierte Videosicherheitslösungen. Separate Ausgänge für Digital- und Analogsignale sind vorhanden.<br><br></p><h3>Funktionen für eine gleichmäßig hohe Bildqualität</h3><p>Diverse Bildoptimierungsfunktionen der HD Analog Bullet Kamera stellen sicher, dass sich Anwender auf eine konstant hohe Videoqualität verlassen können. Wide Dynamic Range (WDR), Gegenlichtkompensation mit BLC und HLC sowie Defog (automatische Korrektur witterungsbedingter Minderungen der Bildqualität) sorgen dafür, dass die Full HD Kamera selbst bei widrigen Lichtverhältnissen und Wetterlagen ein klares Lagebild liefert.<br><br></p><h3>Bei Wind und Wetter rund um die Uhr im Einsatz</h3><p>Das Aluminiumgehäuse entspricht der Schutzklasse IP67 und ist mit einer Heizung ausgestattet, so dass die HD Analog Bullet Kamera für die Außenüberwachung und in einem Temperaturbereich von -30°C?50°C eingesetzt werden kann. Wie alle Kameras der eneo Candid-Serie ist das Modell MPB-72M0550M0A für die schnelle und einfache Installation ausgelegt und kann über eine zentrale Schraube fixiert werden. Darüber hinaus kann sie mit der Anschlussbox AK-4 ergänzt werden, die ebenfalls bei VIDEOR erhältlich ist und die eine professionelle und aufgeräumte Montage ermöglicht.Für den Nachtbetrieb ist die Full HD Kamera schwenkbarem IR-Sperrfilter sowie fünf High Power IR-LEDs (Beleuchtungsreichweite ca. 60m ) versehen.<br><br></p><h3>DSGVO-Funktion und integrierter Bewegungsmelder</h3><p>Die Privatzonenmaskierungsfunktion der HD Analog Bullet Kamera unterstützt Anwender bei der datenschutzkonformen Videoüberwachung. Der integrierte Bewegungsmelder ermöglicht eine automatisierte Überwachung.</p>

Related products

| AK-4 | Camera junction box, for Candid Small ,Candid Medium, Series |
| --- | --- |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| PM-42A | Pole mount adaptor for eneo Candid, Candid S, Candid M, WB1, AK-5, AK-4 |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |