Specifications text

Article number

217277

Type

MPB-72M2713M0A

Short description

1/2.8" HD Camera, Day&Night, 1920x1080, Infrared, WDR, 2.7-13.5mm, 12/24V, IP67

Main Features

1/2.8" 2 Megapixel SONY STARVIS CMOS sensor

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized lens 2.7 ~ 13.5mm

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

IP67, built heater

Easy installation, central locking screw

IR illumination switchable via OSD menu

Other Features (Specifications)

| Audio support | No |
| --- | --- |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Motion detector | No |
| Horizontal angle of view | 100 - 38° |
| Focal length | 2,7 - 13,5 mm |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 100x107,8x277mm |
| Illumination range | ca. 60m |
| Remark | NDAA compliance under Section 889 |
| Watch (62px/m) | 18 m |
| Aperture Range (F) | F1.4 |
| External connections | voltage input,1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC),connection cable approx.100cm |
| Flickerless function | Yes |
| Identify (250px/m) | 4 m |
| Camera shape | Bullet |
| Scope of delivery | manual,drilling template,bag of screws |
| Menu languages | Chinese, English, German, Japanese, Korean |
| Multicast | No |
| Interface Protocols | UTC (HD Signal Format), UTC (HD Signal Format) |
| Signal-to-noise ratio | 50 dB |
| Signal Format | AHD @ 2MP, AHD @ 2MP, Composite (FBAS), CVI @ 2MP, EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP |
| Sunshade canopy | Yes |
| Day/night mode | Automatic,Day,Night |
| Text overlay | available |
| Perceiving (25px/m) | 47 m |
| Integrated wall mount | Yes |
| White balance | Automatic,Preset,Manual |
| Backlight compensation | No |
| Housing type | Outside |
| Housing material | Aluminium |
| Heating | Yes |
| Photosensitivity | 0,08 Lux bei F1,4,40 IRE |
| Low Speed Shutter | No |
| Fan | No |
| Mounting type | Ceiling, Wall |
| Lens Type | Varifocal (motorized) |
| Manufacturer order number | 217277 |
| Protection | IP67 |
| Series | eneo KX-Serie,eneo COAXIZE,eneo Candid Medium |
| Control Interfaces | Coaxitron (HD Protocol) |
| Power supply | 12VDC, 24VAC, 24VAC, 24VAC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -30°C ~ +50°C |
| Vandalism-resistant | No |
| Wide Dynamic Range (WDR) | WDR 113dB |
| Zoomfactor (optical) | 4 |
| Brand | eneo |

Additional Productinformation

Related products

| AK-4 | Camera junction box, for Candid Small ,Candid Medium, Series |
| --- | --- |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor |
| PM-42A | Pole mount adaptor for eneo Candid, Candid S, Candid M, WB1, AK-5, AK-4 |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |