Specifications text

Article number

218643

Type

MPC-54A0003M0A

Short description

1/2,9" HD Camera 2560x1440, Day&Night, AF Zoom, WDR, 3,2-9 mm, Indoor

Main Features

1/2,9" 6,64 Megapixel SONY STARVIS CMOS-Sensor

Resolution max. 2560x1440@25/30fps

Optical 3x auto focus zoom, 3,2~9mm

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD selectable

Digital signal: EX-SDI, HD-SDI selectable

DOL-WDR by triple scan images at 30/25fps &

ROI (Region of Interest) WDR mode

Point of Interest & Smart Motion Zoom

Compact design

Other Features (Specifications)

| Audio support | No |
| --- | --- |
| Recording sensor | CMOS |
| Motion detector | No |
| Horizontal angle of view | 75 - 26° |
| Focal length | 3,2 - 9 mm |
| Chip size | 1/2,9" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 73.5x72x89mm |
| Remark | NDAA compliance under Section 889 |
| Watch (62px/m) | 32 m |
| Aperture Range (F) | F1.6 |
| External connections | voltage input,1x SDI Signal (BNC),1x HD Analog Signal (BNC) |
| Flickerless function | Yes |
| Identify (250px/m) | 6.6 m |
| Camera shape | Zoom cameras |
| Scope of delivery | manual |
| Menu languages | Chinese, English, German, Japanese, Korean |
| Multicast | No |
| Interface Protocols | UTC (HD Signal Format), UTC (HD Signal Format) |
| Signal-to-noise ratio | 50 dB |
| Signal Format | AHD @ 4MP, EX-SDI @ 4MP, HD-SDI @ 2MP, HD-TVI @ 4MP |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night,night-color mode |
| Text overlay | available |
| Perceiving (25px/m) | 80 m |
| Integrated wall mount | No |
| White balance | AUTO,AUTOext,Manual,Preset |
| Backlight compensation | No |
| Housing type | Inside |
| Housing material | Aluminium |
| Heating | Yes |
| Photosensitivity | 0,1 Lux bei F1,6,40 IRE |
| Low Speed Shutter | No |
| Fan | No |
| Lens Type | Auto Focus Zoom |
| Manufacturer order number | 218643 |
| Series | eneo KX-Serie,eneo COAXIZE,eneo Candid Box |
| Control Interfaces | Coaxitron (HD Protocol), RS-485 |
| Power supply | 12VDC, 24VAC, 24VAC, 24VAC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Vandalism-resistant | No |
| Wide Dynamic Range (WDR) | D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, WDR 113dB |
| Zoomfactor (optical) | 3 |
| Brand | eneo |

Additional Productinformation

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| WD-30 | Wall Mount Bracket with Ball Joint, 1/4" Camera Fixing Screw, Pantone Cool Grey 1C |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |