Specifications text

Article number

218643

Type

MPC-54A0003M0A

Short description

1/2,9" HD Camera 2560x1440, Day&Night, AF Zoom, WDR, 3,2-9 mm, Indoor

Main Features

1/2,9" 6,64 Megapixel SONY STARVIS CMOS-Sensor

Resolution max. 2560x1440@25/30fps

Optical 3x auto focus zoom, 3,2~9mm

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD selectable

Digital signal: EX-SDI, HD-SDI selectable

DOL-WDR by triple scan images at 30/25fps &

ROI (Region of Interest) WDR mode

Point of Interest & Smart Motion Zoom

Compact design

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 73.5x72x89mm |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Series | eneo Candid Box, eneo COAXIZE, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 2560x1440 |
| Resolution Pixels | 1280x720, 1920x1080, 2560x1440 |
| System | True Day&Night |
| Sensor size | 1/2.9" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,1 Lux at F1,6, 40 IRE |
| Digital Noise Reduction | yes |
| Lens Type | auto focus zoom |
| Focal Length | 3,2 - 9 mm |
| Iris Control | DC |
| Zoom Factor | 3x |
| Digital Zoom | yes |
| Observe (62 px/m) | 32 - 88 m |
| Recognize (125 px/m) | 14 - 43 m |
| Identify (250 px/m) | 6,6 - 21 m |
| Detect (25px/m) | 80 - 215 m |
| Horizontal Angle of View | 75 - 26° |
| Vertical Angle of View | 41 - 15° |
| MOD (Mininum Object Distance) | 0,5 m |
| Housing | Indoor |
| Color (housing) | Pantone Cool Gray 1C |
| Hosuing material | Aluminium |
| Video outputs | AHD, EX-SDI, HD-SDI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol), RS-485 |
| Analysis functions | Defog, Digital image stabilization |
| Supply voltage | 12VDC, 24VAC |
| Power consumption | 6W |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR, WDR 113dB |
| Motion Detector | yes |
| Signal format | AHD @ 4MP, EX-SDI @ 4MP, HD-SDI (1.485 Gbps), HD-TVI @ 4MP |
| Camera type | Zoom Camera |
| Frame rate | 25/30fps @ 1280x720p, 25/30fps @ 1920x1080p, 25/30fps @ 2560x1440p |
| Aperture range (F) | F1.6 |
| Weight | 300g |
| Manufacturer order number | 218643 |

Additional Productinformation

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| WD-30 | Wall Mount Bracket with Ball Joint, 1/4" Camera Fixing Screw, Pantone Cool Grey 1C |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |