Specifications text

Article number

217279

Type

MPD-62M2713P0A

Short description

1/2.8" HD Dome Fixed, WDR, Day&Night, 2.7-13,5mm, Infrare 1920x1080, 12VDC, Indoor

Main Features

1/2.8" 2 Megapixel SONY STARVIS CMOS sensor

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized vari-focal lens 2.7~13.5mm

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

Wedge adapter included (10° inclination)

Scratch resistant hard coated bubble

Easy installation, 3-Axis gimbal

Other Features (Specifications)

| General |  |
| --- | --- |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 100x116mm |
| Illumination range | ca. 30m |
| Text overlay | available |
| Color (dome) | clear |
| Housing type | Inside |
| Housing material | Plastic |
| Power consumption | 7 W |
| Mounting type | Ceiling, Wall |
| Power supply | 12VDC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 113dB |
| Zoomfactor (optical) | 4 |
| Camera |  |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night |
| White balance | Automatic,Preset,Manual |
| Photosensitivity | 0,08 Lux bei F1,4,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| DORI |  |
| Watch (62px/m) | 18 m |
| Identify (250px/m) | 4 m |
| Perceiving (25px/m) | 47 m |
| Functions |  |
| Digital Noise Reduction (DNR) | Yes |
| Turning range | 165 ° |
| Menu languages | Chinese, English, German, Japanese, Korean |
| Vertical tilt range | 65 °(25 ~ 90°) |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| External connections | voltage input,1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC),connection cable approx.15cm |
| Interface Protocols | UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol) |
| Parts supplied |  |
| Scope of delivery | manual,drilling template,bag of screws |
| Lens |  |
| Horizontal angle of view | 100 - 38° |
| Focal length | 2,8 - 12 mm |
| Aperture Range (F) | F1.4 |
| Lens Type | Varifocal (motorized) |
| Video |  |
| Signal Format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP |
| Product labeling: |  |
| Manufacturer order number | 217279 |
| Series | eneo KX-Serie,eneo Callisto Indoor |
| Brand | eneo |

Additional Productinformation

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-IPM07PRO | 7" (17,8cm) Test Monitor, Touch, IP, HD/3G/EX-SDI, TVI, CVI, AHD, Composite, HDMI |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |