Specifications text

Article number

217279

Type

MPD-62M2713P0A

Short description

1/2.8" HD Dome Fixed, WDR, Day&Night, 2.7-13,5mm, Infrare 1920x1080, 12VDC, Indoor

Main Features

1/2.8" 2 Megapixel SONY STARVIS 2 CMOS sensor

Super low light performance by Sony STARVIS 2 technology

Resolution max. 1920x1080 pixels (Full HD)

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD, CVI and Composite selectable

Digital signal: EX-SDI, HD-SDI selectable

Motorized vari-focal lens 2.7~13.5mm

Removable infrared cut filter

ROI (Region of Interest) WDR mode

Integrated motion detector, defog, privacy zones

Wedge adapter included (10° inclination)

Scratch resistant hard coated bubble

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 100x116mm |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Series | eneo Callisto Indoor, eneo KX-Series |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| Resolution Pixels | 1280x720, 1920x1080, 720x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,08 Lux at F1,4, 40 IRE |
| Digital Noise Reduction | yes |
| Lens Type | Varifocal (motorized) |
| Focal Length | 2,8 - 12 mm |
| Iris Control | DC |
| Zoom Factor | 4x |
| Digital Zoom | no |
| Observe (62 px/m) | 18 - 52 m |
| Recognize (125 px/m) | 8 - 25 m |
| Identify (250 px/m) | 4 - 12 m |
| Detect (25px/m) | 47 - 128 m |
| Horizontal Angle of View | 100 - 38° |
| Vertical Angle of View | 53 - 19° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Indoor |
| Color (housing) | white |
| Color (bubble) | clear |
| Hosuing material | Plastic |
| Heater | no |
| Rotation range | +/-165° |
| Video outputs | AHD, Composite (CVBS), CVI, EX-SDI, HD-SDI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 7W |
| Vandal proof | no |
| Operating temperature | -10°C ~ +50°C |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 113dB |
| Motion Detector | yes |
| Signal format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, EX-SDI (270 Mbps), HD-SDI (1.485 Gbps), HD-TVI @ 2MP |
| Camera type | Dome |
| Frame rate | 25/30fps @ 1920x1080p, 50/60fps @ 1280x720p |
| Aperture range (F) | F1.4 |
| Illumination range | approx. 30m |
| Weight | 260g |
| Manufacturer order number | 217279 |

Additional Productinformation

<h2>eneo Indoor Kamera für die Innenraumüberwachung</h2><p>Beim Fix-Dome MPD-62M2812P0A handelt es sich um eine HD Analog Indoor Kamera, die effektive Videoüberwachung bei ausgezeichneter Preis-Leistung bietet. Der Dome generiert Videobilder mit einer maximalen Auflösung von 1920 x 1080 Pixel (Full HD) und unterstützt über separate Ausgänge die anlogen Videosignale HD-TVI, AHD, CVI und Composite sowie die digitalen Signale EX-SDI, HD-SDI. Das motorisierte Varifokalobjektiv (2,8?12mm) vereinfacht den Installationsprozess ebenso wie der 3-Achsen-Aufhängering der eneo Indoor Kamera. <br><br></p><h3>Automatische Bildoptimierung und Privatzonenschutz </h3><p>Problematische Lichtbedingungen kompensiert die HD Analog Indoor Kamera mittels Bildoptimierungsfunktionen wie WDR, BLC / HLC Gegenlichtkompensation und Defog (automatische Bildkorrektur bei Nebel, Niesel, Smog). Um Anwender bei der Einhaltung der EU-DSGVO zu unterstützen, können sensible Bildbereiche per Privatzonenmaskierung vor Einblick abgeschirmt werden. Außerdem verfügt die eneo Indoor Kamera zur Entlastung des Sicherheitspersonals über einen integrierten Bewegungsmelder. <br><br></p><h3>DORI Distanzen </h3><p>Die optische Leistungsfähigkeit der eneo Indoor Kamera ist in den DORI-Werten dokumentiert: Wahrnehmen (25 px/m) auf 47 bis 128m, Beobachten (62 px/m) auf 18 bis 52m, Erkennen (125 px/m) auf 8 bis 25m und Identifizieren (250 px/m) auf 4 bis 12m. <br><br></p><h3>Keiladapter für Erhöhung des Neigungswinkels </h3><p>Im Lieferumfang enthalten ist ein Keiladapter. Mittels dieses Zubehörteils kann der vertikale Neigungswinkel der HD Analog Indoor Kamera um 10° reduziert werden. Dadurch erhöhen sich die horizontale und vertikale Reichweite der Kamera. Die Kuppel über dem Objektiv ist beidseitig kratzfest beschichtet. <br><br></p><h3>Infrarotbeleuchtung an Bord </h3><p>Die eneo Indoor Kamera kommt mit einem schwenkbaren IR-Sperrfilter und integrierter Infrarotbeleuchtung. Diese besteht aus 2 High Power LEDs, die eine Reichweite von 30m haben. Damit ist MPD-62M2812P0A bestens für die Überwachung rund um die Uhr ausgestattet.</p>

Related products

| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| SC-04MHD | Quad Splitter, 1920x1080, Composite, AHD, TVI, CVI Input Composite, HDMI, VGA Output |
| SC-IPM07PRO | 7" (17,8cm) Test Monitor, Touch, IP, HD/3G/EX-SDI, TVI, CVI, AHD, Composite, HDMI |
| SC-MFM07HD | Test Monitor, Multiformat, for Service, HD-TVI/CVI/AHD, HD-SDI, EX-SDI, FBAS, HDMI |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |