Occupancy Monitoring

# Document Control

## Information

| Document Control |  |
| --- | --- |
| Document Name: | People Counting Deployment.230620.docx |
| Pre Sales Support: | Ian Cook |
|  | i.cook@hanwha.com |
|  | +44 (0) 7824 993 520 |
| Date: | 25/06/2020 |

## Revision Status

| Name | Date | Version | Cause |
| --- | --- | --- | --- |
| Ian Cook | 23/06/2020 | V1.1 | Initial Release |
| Ian Cook | 25/06/2020 | V1.3 | Added Decoder & added People Counting line Setup |

Table of Contents

1	Document Control	2

1.1	Information	2

1.2	Revision Status	2

2	Purpose of Document	4

2.1	AI Camera Overview	4

2.2	Solution description	5

2.2.1	Minimum PC requirements	5

2.2.2	Camera Output	6

2.2.3	Setup Examples	6

3	Camera Setup	8

3.1	Setting up AI Object Classification	10

3.2	Setting up People Counting	11

4	App Setup	12

4.1	CGI Commands for Camera output	14

4.2	Relay control	15

4.3	Audio Announcement	16

5	Web Page	17

6	Using an SPD-151 Decoder	17

6.1	Setting up the SPD-151	18

6.2	Decoder Notes:	20

# Purpose of Document

This document has been created by Hanwha Techwin to provide an overview of the People Occupancy Monitoring Application that is used along with Hanwha Techwin’s AI cameras.

## AI Camera Overview

The new Artificial Intelligence cameras offer a number of significant improvements with regards to Video Analytics, improving reliable detection of logical events of specified conditions from the camera’s video.

The P Series AI cameras are 4K/8MP Cameras, with the dome cameras and Bullet cameras having a built in motorised zoom lens of 4.5-10mm, with 30meter IR illumination. The PNB-A9001 will use a Megapixel rated manual zoom lens.

The AI cameras are as follows:

The AI Cameras

- Classified object type : Person/Face/Vehicle/License plate with attributes, Best Shot per object

- Analytics events based on AI engine : Object detection, Directional detection, Virtual line, Enter/Exit, Loitering, Appear/Disappear, Digital auto tracking,

- Analytics events : Defocus detection, Motion detection, Tampering, Audio detection, Sound classification, Shock detection

Business Intelligence features based on AI engine, classified object type: People counting, Queue management, Heat map:

- Person 	Gender: Female / Male

- Upper/Lower (colour): Black / Gray / White / Red / Orange / Yellow / Green / Blue / Purple (up to 2 colour at same time)

- Bag : Wear Bag or Not

- Vehicle	Type: Car(sedan/SUV/Van) / Bus / Truck / Motorcycle / Bicycle

- Colour: Black / Gray / White / Red / Orange / Yellow / Green / Blue / Purple (up to 2 colour at same time)

- Face Gender: Female / Male

- Age: Child / Young / Middle / Old

- Glasses: Wear Glasses or Not

## Solution description

Unlike traditional people counting cameras that require top down view, the AI cameras can be used in perspective view, and therefor can be used to replace existing IP Cameras, and achieve operationally similar Field of Views, but in addition, giving the additional functionality of the AI cameras.

Top Down (Standard Cameras)   			Perspective View (AI Cameras)

The Occupancy Monitoring solution relies on 1 Master AI Camera, and up to 7 Slave cameras. The Master Camera is loaded with an Open Application cap program using the master cameras web page.

The App aggregates multiple counts from multiple cameras, and will serve up a web page to display the output.  In order to display the App, a Windows PC is required for monitor display, multiple PC’s and Monitors can be used. The Specification of the PC is any Windows 10 based PC, with the following minimum specification:

### Minimum PC requirements

  - CPU: Intel Atom

  - RAM: 4GB

  - Storage: 64GB

  - OS: Windows or Linux

  - Browser: Microsoft Edge, Chrome and other browsers that supports HTML5

Hanwha Techwin have tested on a Mini Stick PC, and an Intel NUC PC and both have been proven to work, so any PC specification will be sufficient. It should be noted, that it is recommended to change the BIOS setting to allow Automatic restart on Power.

### Camera Output

All Hanwha cameras have a digital output, also known as a TTL output. In order to use this output with 3rd party equipment, there is a need to supply a Transistorised relay. This Relay uses a 12vDC Power Supply, and allows the Camera TTL output to change the Relay state.

The following Relay / PSU is recommended:

| Title | Description | Link |
| --- | --- | --- |
| Digital Input Output | 2 Pole Transistorised Relay for connection to Camera TTL Output | Link |
| PSU | 12v 2Amp Power Supply for Relay | Link |

The Camera relay is controlled by the use of two command like actions (Called CGI Commands) – one for when the system is over occupancy levels, the second for normal state. These commands simply tell a specific cameras output to change state according to the count information, please refer to section 4.1 CGI Commands for Camera output

### Setup Examples

The following examples are provided for reference:

# Camera Setup

Using Wisenet Device Manager, or Web Browser you would log into the camera and setup as you normally would for general surveillance, such as changing the admin credentials, IP Address and simple focus, you will need to adjust the Field of View for the AI People Counting.

The camera need to be able to see (in clear view) doors you wish to monitor, therefore you will need to position the camera without obstructions (such as marketing, or stock) – The AI camera doesn’t need to be highly zoomed into the doors, but it must be able to see people traversing through the doors.

Using the Wisenet Toolbox Plus web tool, you can establish the Fov based on tilt angle, zoom and height of the camera. You can access this tool here: https://www.hanwha-security.com/wisenettoolbox_plus/index.html#!/en/home

Search for one of the supported AI cameras, and then click the FoV section under the camera to open up the tool:

One the tool is open, you can adjust your settings to verify the FoV is suitable:

The AI cameras are far more accurate than traditional Video Analytics cameras (those based on Machine learning) – so you are able to widen the FoV much further that would previously have been possible. A view similar to the below would be suitable:

Once you have set up the FoV (Which may take further adjustments, you need to enable the AI Object Classification.

## Setting up AI Object Classification

Once the general setup of the camera has been completed, go to Setup, Analytics, Object Detection, and Enable the Person object, set the minimum duration to 1-2 seconds (you can adjust this later if required) – and hit Apply:

Once this is completed you will need to define the “Common” tab, this specified the minimum (blue box) and Maximum size (Red Box) for that scene. Hit apply when you are happy you have adjusted these.

## Setting up People Counting

Once the AI Object Classification has been set up, there is a need to configure People Counting, which is under Statistics / People Counting / Setup

Enable the People Counting, then use 1 or 2 lines depending on one or two entrances – each line will count both directions (in/Out). You will need to test the line / Human passing a few times to ensure the position works for your site.

You can also use the cameras only daily/ weekly reporting from the search tab:

# App Setup

The Occupancy app is distributed with this document, and is called “occupancy_monitoring.cap” – we can supply this separated, or simply copy, paste the below file to your PC:

Please update the AI Camera Firmware to the latest version – which is currently 1.41.02_20200604. AI Camera Firmware can be downloaded from this link, and searching for the specific model you are using: https://docu.hanwha-security.com/#/contents

Using the Setup page of the camera, go to – Open Platform section on the right hand side of the web page.

- Browse to where you saved the cap file on your local PC

- Install the app

- Confirm the “Ok” to install the app, Confirm OK, to accept the app has been installed.

One the installation of the app has been completed, you will need to enable “Auto Start”, and actually start the App.

Once “Go App” is clicked, a new webpage will open. This is where you configure your Occupancy Monitoring app:

## CGI Commands for Camera output

In order to make the system as flexible as possible, it is not necessarily the master camera that has to be wired back to in order to use the camera output control.  If the Master Camera is being used for Relay control, you can simply select the tick option (shown above), otherwise using the HTTP Tab, you can add in custom HTTP Commands / CGI for other Hanwha Cameras, or 3rd party devices like the Advantech ADAM 6050 IP I/O Device.  It is possible to have any Hanwha camera be used as the TTL output for connecting to a relay.

Simply replace the [IP] section of the below command (after the http://) – with the ip address of the camera where you wish to use the output:

Turn on : http://[IP]/stw-cgi/io.cgi?msubmenu=alarmoutput&action=control&AlarmOutput.1.State=On&AlarmOutput.1.ManualDuration=Always

Turn off: http://[IP]/stw-cgi/io.cgi?msubmenu=alarmoutput&action=control&AlarmOutput.1.State=Off

## Relay control

Once the CGI commands are in, there will be a need to supply a 3rd party relay, PSU and other elements like traffic light system.  The following diagram shows how to wire up the relay for the purpose of connecting other devices

If the relay is 12vDC with 50ma coil or less, the AI Camera can drive this output, or otherwise, please use a Transistorised Relay and additional PSU for the Relay power.

## Audio Announcement

The Hanwha P Series AI Cameras have an ability to output pre-recorded Audio clips to amplified Speakers connected to the camera. Please refer to the specific camera manual for further details on the audio output.

This audio clip is able to be announced based on the presence of people in the area – giving them instruction to look at the monitor screen, proceed if Green, or wait for available space if the display is Red.  Wave files must be used for this function, with a maximum of 5 files, and a total file size of less than 3MB. Wav files can be easily converted to Mono, and the quality reduced if required. Audio clips are uploaded under the Settings / Audio Setup / Audio Clip:

In order to get the Audio Clip to play back based on Persons being in the scene – you need to set up the AI Object detection, select “Persons”, select the Audio Clip you uploaded, and set the schedule if desired:

# Web Page

It is recommended to have google chrome for the default web browser, however Microsoft Edge, Firefox and Safari would also work.

Once you click login on the setup page the following two web pages will be displayed according to the count, green being OK to come in, and Red to stop and wait.

There is a hidden Menu (Mouse hover at the Left or Right hand side of the browser windows for 2-3 seconds)  This allows you to apply a manual adjustment to the count, go back to the settings, or Reset the Count.

# Using an SPD-151 Decoder

When there is a need to use an embedded appliance opposed to a PC, The Hanwha Techwin SPD-151 Decoder can be used as the source for a Display Monitor.

The SPD-151 is a low cost decoder, Powered by PoE or 12vDC power, and supports HDMI and VGA monitors. Limited CVBS Output is also possible, although not recommended for this type of deployment. Configuration is carried out via Web Browser.

Further Information on the SPD-151 is available here: https://www.hanwha-security.eu/business-security-products/spd-151/

## Setting up the SPD-151

In order to get the Master camera and the SPD-151 communicating, there are some basic steps to be set up on the decoder.

- As with all Hanwha Techwin equipment, the SPD-151 is best configured by searching for it using our Wisenet Device manager – which is available here: https://www.hanwha-security.eu/wp-content/uploads/2020/02/Device-Manager-SW_Device-Manager_200124_Common_v.2.3.5.zip

- Once the SPD-151 has been discovered, log in, and change the default Administrator Password credentials.

- In order to get the SPD-151 working, you will need to define a layout – so using the Video Wall tab:

  - Create a 1x1 Layout.

  - Drag either the HDMI, or VGA monitor onto the layout.

  - There is no need to Save this setting.

- Once the Video Wall has been configured, the next step is to add in the RTSP (Video Stream) as a Camera.

- Using the Layout Tab, click the + Symbol on the left hand side, under “Camera”

A Pop up will be displayed:

- Select “Manual”

- Select “RTSP”

- Enter the RTSP String – replacing the [IPAddress] with that of the Master Camera (i.e. 192.168.1.100)

rtsp://[IPAdress]:3128/OccupancyMonitoring/media.smp

- Enter Admin Credentials

- Click Save

- You will get a pop up stating that access is successful, do you want to register, click Yes

- Finally, you will see the new RTSP “Camera” in the list of devices:

- Drag the RTSP “Camera” onto the Layout

- Click Save

## Decoder Notes:

- The RTSP feed remains persistent, and doesn’t need any further configuration. It will display the count, as long as the App is running on the Master Camera.

- Because of the way the decoder works, there is no scrolling / Static message possible.

- Because it is an MJPEG feed, the customised background Colours are not shown on the decoder, they are however clearly Red / Green Displays:

About Hanwha Techwin Europe

Hanwha Techwin Europe, formerly Samsung Techwin Europe and part of Samsung Group, offers robust and reliable video surveillance products designed to meet the current and future needs of security professionals.

Our cameras, video recorders and other IP network devices are built to exacting standards which reflect the heritage of precision engineering associated with the Samsung brand.

What separates Hanwha Techwin Europe from the crowd is the long list of innovative features built into our products which collectively deliver tangible benefits and outstanding value to customers. Much of the technology behind these features is unique to Hanwha Techwin Europe.

Hanwha Techwin Europe has been among the first to recognise the value of providing end-users with the option to run software solutions such as video analytics at the edge i.e. within the camera.

One of the keys strengths of our Open Platform WiseNet 5 DSP chipsets, and forthcoming Wisenet 7 chipsets, which are at the heart of our latest generation of high definition cameras, providing customers with complete freedom to choose the perfect combination of software Applications, as well as VMS (video management software) such as Hanwha Techwin’s “Smart Security Manager”, or Wisenet Wave to best match their individual operational requirements.

Offering ‘Best in Class’ products is only part of the process of delivering a complete video surveillance solution; every one of Hanwha Techwin Europe’s products, many of which have been highly rated in independent product tests, is backed-up by unbeatable pre-and post-sales support. Training is at the heart of a range of support services which also includes system design, technical support and a full three-year warranty.

Hanwha Techwin Europe’s security solutions are widely accessible across Europe via an extensive network of distributors.

About Hanwha Group

Hanwha Group, founded in 1952, is one of the Top-Ten business enterprises in South Korea and a “FORTUNE Global 500” company. Hanwha Group has 52 domestic affiliates and 146 global networks in three major sectors: manufacturing and construction, finance, and services and leisure.

With more than 60 years’ track record of industrial leadership, Hanwha’s manufacturing and construction businesses encompass a broad range of fields from basic chemicals and advanced materials to property development and total solar energy solutions. The finance network, covering banking, insurance, asset management and securities, is the second largest non-bank finance group in South Korea. The services and leisure sector offers premium lifestyle services with retail and resort businesses.

Hanwha Techwin Europe Limited

Heriot House, Heriot Road, Chertsey, Surrey, KT16 9DT

United Kingdom

www.hanwha-security.eu